GESTURE-CONTROLLED SMART

GAS, FIRE SAFETY &

VENTILATION SYSTEM

Using ESP32 + MQ135 + Flame Sensor + MPU6050

Complete Project Documentation

Prepared For: Project Submission & Presentation

# SECTION 1: Simple Explanation (Beginner Friendly)

## 1.1 What Does This System Do?

This project is a smart safety system that uses an ESP32 microcontroller to monitor your environment for dangerous gas leaks and fire, and then automatically responds to protect you. Think of it as a smart guardian for your home or lab.

What makes this system unique is gesture control. You don't need to touch any buttons or knobs. Instead, you tilt or shake a small sensor module (MPU6050) to turn the system ON/OFF, reset alarms, or trigger emergency mode — just like controlling a device with hand gestures!

Think of it like this: You're cooking and gas starts leaking. The system detects it, turns on the exhaust fan automatically, flashes a red LED, and beeps an alarm — all without you having to do anything!

## 1.2 Why Are Both Gas AND Flame Detection Important?

Gas and fire are two separate but related dangers:

- Gas detection (MQ135): Gas can build up invisibly and silently. You might not smell it until it's too late. It is also explosive — even a small spark can cause a blast.

- Flame detection: A small fire can spread very fast. Early flame detection gives precious seconds to react before it becomes a large fire.

- Together: A gas leak CAN start a fire. By detecting BOTH, the system catches the danger at two stages — before ignition (gas) AND after ignition (flame).

## 1.3 Real-Life Use Cases

| Use Case | How This System Helps |
| --- | --- |
| Home Kitchen | Detects LPG/cooking gas leaks. Turns on exhaust fan automatically. |
| Laboratory | Detects chemical vapors or solvent fumes. Triggers ventilation. |
| Industrial Plant | Monitors for toxic gas levels, triggers emergency shutdown. |
| Server Room | Detects overheating/burning components via smoke detection. |
| Garage / Workshop | Detects CO or fuel vapors from vehicles or generators. |
| Hospital / ICU | Monitors oxygen/gas lines for leaks — critical patient safety. |

# SECTION 2: Detailed Working Principle

## 2.1 How MQ135 Detects Gas / Smoke

The MQ135 is a chemo-resistive gas sensor. Inside it, there is a small ceramic tube coated with a sensitive metal oxide material (tin dioxide, SnO2). Here is how it works step by step:

- A small internal heater warms the sensor element to its operating temperature (~200–300°C).

- In clean air, oxygen ions attach to the SnO2 surface, creating a high electrical resistance.

- When gas or smoke molecules (like CO2, NH3, benzene, alcohol, smoke) contact the heated surface, they react with the oxygen ions.

- This reaction frees electrons, reducing the resistance of the sensor dramatically.

- The ESP32 reads this resistance change as an analog voltage on GPIO34.

- A higher voltage reading (higher analog value 0–4095) means more gas present.

Important: MQ135 needs a warm-up time of 20–30 seconds after power-on before giving stable readings. Always wait before trusting the sensor readings!

## 2.2 How the Flame Sensor Works

The flame sensor module contains a photodiode that is specially sensitive to infrared (IR) radiation in the 760nm–1100nm wavelength range — the same wavelength that burning flames emit strongly.

- When no flame is present: The photodiode sees only ambient light. Its output remains HIGH (digital 1).

- When a flame is present: The IR radiation from the fire hits the photodiode. Current flows, causing the output to go LOW (digital 0).

- The ESP32 reads this as a digital input on GPIO35.

- Since the sensor is active-LOW, we check: if (flameVal == LOW) → fire detected.

The flame sensor typically detects a candle flame from up to 80cm away, and a larger fire from several meters.

## 2.3 How MPU6050 Detects Gestures

The MPU6050 is a 6-axis Inertial Measurement Unit (IMU). It combines a 3-axis accelerometer and a 3-axis gyroscope on a single chip.

The accelerometer measures acceleration along three axes (ax, ay, az) in units of 'g' (gravity). At rest on a flat surface, az ≈ +16384 (which equals +1g due to gravity), and ax, ay ≈ 0.

| Gesture | Axis Behaviour | Detection Logic | Action Taken |
| --- | --- | --- | --- |
| Tilt Right | ax becomes strongly positive (>8000) | ax > TILT_THRESHOLD | System turns ON |
| Tilt Left | ax becomes strongly negative (<-8000) | ax < -TILT_THRESHOLD | System turns OFF |
| Tilt Up | az becomes strongly positive, ay near 0 | az > TILT_THRESHOLD | Alarm Reset |
| Shake | All axes change rapidly and suddenly | delta > SHAKE_THRESHOLD | Emergency Mode |

## 2.4 How the ESP32 Processes Everything

The ESP32 acts as the brain of the system. In every loop iteration:

- Read MPU6050 via I2C → Detect gesture → Update system state (ON/OFF/Emergency).

- If system is OFF, skip all sensor checks and keep outputs OFF.

- Read MQ135 analog value → Compare with threshold → Set gasDetected flag.

- Read Flame sensor digital value → Set fireDetected flag.

- Apply priority logic: Emergency > Fire > Gas > Safe.

- Update outputs: LED colours, buzzer pattern, relay (fan), LCD message.

## 2.5 How Outputs Respond

- Relay: Controls a fan/exhaust system. Activated HIGH when danger is detected. Turns OFF in safe condition.

- Buzzer: Produces different beep patterns (slow, fast, continuous) based on danger level.

- Red LED: Glows during any danger condition (gas, fire, emergency).

- Green LED: Glows only in safe condition when system is ON.

- LCD: Shows status messages and live gas sensor values for easy monitoring.

# SECTION 3: System Block Diagram

The following text-based block diagram shows the flow of data and control in the system:

┌─────────────────────────────────────────────────────────────────┐

│ POWER SUPPLY (5V USB) │

└───────────────────────────┬─────────────────────────────────────┘

│

┌───────────────────────────▼─────────────────────────────────────┐

│ INPUT SENSORS │

│ │

│ ┌────────────┐ ┌────────────┐ ┌────────────────────────┐ │

│ │ MQ135 │ │ FLAME │ │ MPU6050 │ │

│ │ Gas Sensor │ │ Sensor │ │ Accelerometer+Gyro │ │

│ │ (Analog) │ │ (Digital) │ │ (I2C: SDA/SCL) │ │

│ └─────┬──────┘ └─────┬──────┘ └──────────┬─────────────┘ │

└────────┼───────────────┼──────────────────────┼─────────────────┘

│ │ │

│ GPIO34 │ GPIO35 │ I2C (0x68)

│ │ │

┌────────▼───────────────▼──────────────────────▼─────────────────┐

│ │

│ ESP32 (38-pin) │

│ [Central Processing Unit] │

│ │

│ • Reads all sensors │

│ • Runs gesture detection algorithm │

│ • Applies priority safety logic │

│ • Controls all output devices │

│ │

└────────┬───────────────┬──────────────────────┬─────────────────┘

│ │ │

┌────────▼───────────────▼──────────────────────▼─────────────────┐

│ OUTPUT DEVICES │

│ │

│ ┌─────────┐ ┌──────────┐ ┌──────────┐ ┌──────────────────┐ │

│ │ Relay │ │ Buzzer │ │ Red LED │ │ 16x2 I2C LCD │ │

│ │ GPIO26 │ │ GPIO25 │ │ GPIO27 │ │ I2C Addr 0x27 │ │

│ │(Fan ON) │ │(Beep Pat)│ │ Danger │ │ Status Display │ │

│ └────┬────┘ └──────────┘ └──────────┘ └──────────────────┘ │

│ │ ┌──────────┐ │

│ │ │ Green LED│ GPIO14 (Safe) │

│ ┌────▼─────┐ └──────────┘ │

│ │Exhaust │ │

│ │Fan/Pump │ │

│ └──────────┘ │

└──────────────────────────────────────────────────────────────────┘

# SECTION 4: Complete Circuit Connections

## 4.1 Complete Pin Connection Table

| Component | Component Pin | ESP32 Pin / Connection | Notes |
| --- | --- | --- | --- |
| MQ135 Gas Sensor | VCC | 3.3V or 5V | Use 5V for better sensitivity |
| MQ135 Gas Sensor | GND | GND | Common ground |
| MQ135 Gas Sensor | A0 (Analog Out) | GPIO34 | ADC1 channel, 0–3.3V only |
| MQ135 Gas Sensor | D0 (Digital Out) | Not connected | We use analog for precision |
| Flame Sensor | VCC | 3.3V |  |
| Flame Sensor | GND | GND | Common ground |
| Flame Sensor | D0 (Digital Out) | GPIO35 | Active LOW output |
| MPU6050 | VCC | 3.3V | Do NOT connect to 5V! |
| MPU6050 | GND | GND | Common ground |
| MPU6050 | SDA | GPIO21 | I2C Data — shared bus |
| MPU6050 | SCL | GPIO22 | I2C Clock — shared bus |
| MPU6050 | AD0 | GND | Sets I2C address to 0x68 |
| MPU6050 | INT | Not connected | Not used in this project |
| 16x2 LCD (I2C) | VCC | 5V | LCD needs 5V for backlight |
| 16x2 LCD (I2C) | GND | GND | Common ground |
| 16x2 LCD (I2C) | SDA | GPIO21 | Shared I2C bus with MPU6050 |
| 16x2 LCD (I2C) | SCL | GPIO22 | Shared I2C bus with MPU6050 |
| Buzzer | Positive (+) | GPIO25 | Through 220Ω resistor |
| Buzzer | Negative (–) | GND |  |
| Relay Module | VCC | 5V | Relay coil needs 5V |
| Relay Module | GND | GND | Common ground |
| Relay Module | IN (Signal) | GPIO26 | Control signal from ESP32 |
| Relay Module | COM | Fan power positive | Fan/exhaust connection |
| Relay Module | NO (Normally Open) | Fan load wire | Fan turns ON when relay triggers |
| Red LED | Anode (+) | GPIO27 via 220Ω resistor | 220Ω limits current to ~15mA |
| Red LED | Cathode (–) | GND |  |
| Green LED | Anode (+) | GPIO14 via 220Ω resistor | 220Ω limits current to ~15mA |
| Green LED | Cathode (–) | GND |  |

## 4.2 Step-by-Step Wiring Guide

### Step 1: Power and Ground Setup

- Connect ESP32 5V pin → Breadboard positive (+) power rail

- Connect ESP32 3.3V pin → Second breadboard power rail (for 3.3V components)

- Connect ESP32 GND → Breadboard negative (–) ground rail

- All component GND pins connect to the ground rail

### Step 2: I2C Bus Wiring (shared by MPU6050 and LCD)

- Connect ESP32 GPIO21 → Breadboard row A (this is the SDA line)

- Connect ESP32 GPIO22 → Breadboard row B (this is the SCL line)

- Connect MPU6050 SDA → Same row as GPIO21 (row A)

- Connect MPU6050 SCL → Same row as GPIO22 (row B)

- Connect LCD SDA → Same row A

- Connect LCD SCL → Same row B

- Connect MPU6050 AD0 → GND (sets address 0x68)

Both MPU6050 (address 0x68) and LCD (address 0x27) share the same two I2C wires. This is perfectly fine — I2C supports multiple devices on the same bus. They talk one at a time.

### Step 3: MQ135 Gas Sensor

- Connect MQ135 VCC → 5V rail

- Connect MQ135 GND → GND rail

- Connect MQ135 A0 (analog output) → ESP32 GPIO34

IMPORTANT: GPIO34 on ESP32 is input-only. It works perfectly for reading analog sensors. Make sure you connect to the A0 pin of MQ135, not D0.

### Step 4: Flame Sensor

- Connect Flame Sensor VCC → 3.3V rail

- Connect Flame Sensor GND → GND rail

- Connect Flame Sensor D0 (digital out) → ESP32 GPIO35

### Step 5: LEDs (with 220Ω resistors)

- Insert a 220Ω resistor in series with each LED

- Red LED: ESP32 GPIO27 → 220Ω resistor → Red LED anode → LED cathode → GND

- Green LED: ESP32 GPIO14 → 220Ω resistor → Green LED anode → LED cathode → GND

Always use a resistor with LEDs. Without it, too much current flows and burns out the LED (or damages the ESP32 GPIO pin). 220Ω gives a safe ~15mA current.

### Step 6: Buzzer

- Connect Buzzer positive (+) → 220Ω resistor → ESP32 GPIO25

- Connect Buzzer negative (–) → GND

### Step 7: Relay Module

(you can add if you want I have not used it)

# SECTION 5: Arduino Code requirements

Required Libraries: Install these via Arduino IDE → Sketch → Include Library → Manage Libraries: 1. LiquidCrystal_I2C (by Frank de Brabander) 2. MPU6050 (by Electronic Cats or jrowberg)

Also set: Tools → Board → ESP32 Dev Module | Upload Speed: 115200 | Flash Size: 4MB

# SECTION 6: Gesture Logic Explanation

## 6.1 Understanding Accelerometer Values (ax, ay, az)

The MPU6050 accelerometer returns raw 16-bit signed integers for each axis. With default sensitivity (±2g range), the values are:

| Value | Meaning | Typical Range |
| --- | --- | --- |
| ax | Acceleration along X-axis (left-right tilt) | –16384 to +16384 |
| ay | Acceleration along Y-axis (forward-back tilt) | –16384 to +16384 |
| az | Acceleration along Z-axis (up-down, gravity) | –16384 to +16384 |
| +16384 | Equals +1g (full gravity in that direction) | When chip is flat: az ≈ +16384 |
| 0 | No acceleration in that direction | When flat: ax=0, ay=0 |

## 6.2 How Tilt is Detected

When you tilt the MPU6050 module (which sits on your hand or a platform), gravity shifts its apparent direction:

- Flat (normal): ax=0, ay=0, az=+16384

- Tilt Right 90°: ax=+16384, ay=0, az=0 (gravity now along X-axis)

- Tilt Left 90°: ax=–16384, ay=0, az=0

- Tilt Up (face up): az goes positive and large

In our code, we use a threshold of 8000 (roughly 0.5g). This means a tilt of about 30° or more is enough to trigger a gesture — comfortable and reliable.

## 6.3 How Shaking is Detected

Shaking causes rapid changes across ALL axes. We detect this by comparing current readings with the previous readings:

- delta_ax = |ax_now – ax_prev|

- delta_ay = |ay_now – ay_prev|

- If ANY delta exceeds SHAKE_THRESHOLD (15000), it's a shake.

Shaking creates sudden, violent changes (>15000 raw units) that are easily distinguishable from normal tilting.

## 6.4 How to Adjust Sensitivity

| Parameter | Default Value | Increase to… | Decrease to… |
| --- | --- | --- | --- |
| TILT_THRESHOLD | 8000 | Require more extreme tilt (harder to trigger) | Allow smaller tilt (easier to trigger) |
| SHAKE_THRESHOLD | 15000 | Require more violent shaking | Detect gentler shaking |
| loop() delay | 200ms | Make system less responsive | Make system more responsive |

# SECTION 7: System Logic Table

| System Condition | Green LED | Red LED | Buzzer Pattern | Relay (Fan) | LCD Display |
| --- | --- | --- | --- | --- | --- |
| System OFF (tilt left) | OFF | OFF | Silent | OFF | SYSTEM OFF / Tilt Right=ON |
| Safe – All Clear | ON | OFF | Silent | OFF | STATUS: SAFE + Gas value |
| Gas Detected | OFF | ON | Slow beep (500ms) | ON | ** GAS ALERT ** + Level |
| Fire Detected | OFF | ON | Continuous ON | ON | ** FIRE ALERT ** / FIRE DETECTED! |
| Emergency (Shake) | OFF | ON | Fast beep (100ms) | ON | !!! EMERGENCY !!! / Tilt UP=Reset |
| Alarm Reset (tilt up) | ON | OFF | Silent | OFF | ALARM RESET → then SAFE |
| System ON (tilt right) | ON | OFF | Silent | OFF | SYSTEM ON → then SAFE |

# SECTION 8: Algorithm / Flowchart (Text Form)

START

│

▼

[INITIALIZE]

• Set all pins as INPUT/OUTPUT

• Init I2C (GPIO21=SDA, GPIO22=SCL)

• Init LCD → Show welcome message

• Init MPU6050 → Test connection

• Green LED ON, all others OFF

│

▼

┌─────────────────────────────────────┐ ← MAIN LOOP (repeats forever)

│ │

│ [READ MPU6050] │

│ Get ax, ay, az │

│ │ │

│ ▼ │

│ [GESTURE CHECK] │

│ Calculate delta from prev values │

│ │ │

│ Is delta > SHAKE_THRESHOLD? │

│ YES → Set emergencyMode = TRUE │

│ NO → Check tilt direction │

│ │ │

│ ax > TILT_THRESHOLD? │

│ YES → systemOn = TRUE │

│ │ │

│ ax < -TILT_THRESHOLD? │

│ YES → systemOn = FALSE │

│ → All outputs OFF │

│ → LCD: SYSTEM OFF │

│ → RETURN (skip rest) │

│ │ │

│ az > TILT_THRESHOLD? │

│ YES → Reset emergency+alarms │

│ │ │

│ Is systemOn == FALSE? │

│ YES → RETURN (skip sensors) │

│ │ │

│ [READ SENSORS] │

│ gasValue = analogRead(GPIO34) │

│ flameVal = digitalRead(GPIO35) │

│ │ │

│ [PRIORITY LOGIC] │

│ │ │

│ emergencyMode == TRUE? │

│ YES → Red ON, Fan ON, Fast Beep │

│ LCD: EMERGENCY │

│ RETURN │

│ │ │

│ flameVal == LOW? (fire detected) │

│ YES → Red ON, Fan ON, Continuous │

│ Beep, LCD: FIRE ALERT │

│ RETURN │

│ │ │

│ gasValue > GAS_THRESHOLD? │

│ YES → Red ON, Fan ON, Slow Beep │

│ LCD: GAS ALERT + Level │

│ RETURN │

│ │ │

│ [ALL CLEAR — SAFE] │

│ Green ON, Red OFF │

│ Fan OFF, Buzzer OFF │

│ LCD: STATUS: SAFE + gas value │

│ │ │

│ delay(200ms) │

└─────────────────────────────────────┘

LOOP BACK TO TOP

# SECTION 9: Testing & Calibration

## 9.1 Testing the MQ135 Gas Sensor

- Power on the circuit and wait 30 seconds for MQ135 to warm up. Initial readings will be unstable — this is normal.

- Open Arduino IDE Serial Monitor (115200 baud). Watch the gas value printed in the loop.

- In clean air, the value should stabilize between 200–400 (approximate). Note your clean-air baseline.

- To test: Hold a lit matchstick or a small piece of burned paper (safely) near the sensor. You should see the value jump above 600.

- Adjust GAS_THRESHOLD based on your baseline: set it about 150–200 above your clean-air reading.

Safety Tip: Never test with actual LPG/gas from a cylinder. Use a matchstick, alcohol wipe, or a hand sanitizer bottle briefly opened near the sensor. Keep ventilation open!

## 9.2 Testing the Flame Sensor

- Point the flame sensor toward a normal candle flame from 30cm away. The digital output should go LOW.

- Watch Serial Monitor: 'Flame: 0' means fire detected, 'Flame: 1' means no fire.

- There is usually a small potentiometer (blue trimmer) on the module. Turn it to adjust sensitivity.

- Turning clockwise: increases sensitivity (detects flame from farther away).

- Turning counterclockwise: decreases sensitivity (reduces false triggers from sunlight).

Avoid pointing the flame sensor directly at bright sunlight or fluorescent lights — the IR content can trigger false alarms. Adjust the potentiometer if needed.

## 9.3 Testing MPU6050 Gestures

- Upload the code. Open Serial Monitor.

- Keep the ESP32+MPU6050 flat on a table. You should see no gesture messages.

- Tilt the board firmly to the RIGHT. You should see 'GESTURE: TILT RIGHT -> System ON'.

- Tilt the board firmly to the LEFT. You should see 'GESTURE: TILT LEFT -> System OFF'.

- Tilt the board so it faces up (rotate forward). You should see 'GESTURE: TILT UP -> Alarm Reset'.

- Shake the board vigorously. You should see 'GESTURE: SHAKE -> Emergency Mode!'.

- If gestures don't trigger: reduce TILT_THRESHOLD (try 6000). If false triggers occur: increase TILT_THRESHOLD (try 10000).

# SECTION 10: Advantages and Limitations

## 10.1 Advantages

| Advantage | Description |
| --- | --- |
| Touchless Control | Gesture-based ON/OFF and reset — no physical switches needed. Useful in hazardous/contaminated environments. |
| Dual Hazard Detection | Detects both gas leaks AND fire independently — two layers of safety. |
| Automatic Response | Relay-controlled fan activates automatically — no human intervention needed. |
| Visual + Audio Alerts | Multiple alert methods (LED, buzzer, LCD) ensure the alarm is noticed. |
| Distinct Alert Patterns | Different buzzer patterns for gas vs fire vs emergency — easy to identify hazard type. |
| Low Cost | All components are inexpensive and widely available. Total project cost under ₹800–1200. |
| ESP32 Power | ESP32 has built-in WiFi/Bluetooth — can be upgraded to send mobile alerts easily. |
| Real-time Display | LCD shows live gas values, allowing users to monitor trends over time. |

## 10.2 Limitations

| Limitation | Description |
| --- | --- |
| MQ135 Warm-up Time | Needs 30–60 seconds to warm up after power-on. False readings during this period. |
| No Specificity | MQ135 is a broad-spectrum sensor. It cannot distinguish between different gas types (LPG vs CO vs NH3). |
| Flame Sensor Range | Limited detection range (~80cm for candle). Large fires detected from farther, but small sparks may be missed. |
| No IoT Integration | This version has no WiFi notification. If the user is asleep/away, they may not hear the buzzer. |
| Gesture Accuracy | Gesture detection can occasionally false-trigger if the board is bumped or moved during transport. |
| No Data Logging | The system does not store historical readings. There is no way to review past events. |
| Single Relay | Only one exhaust fan is controlled. Real industrial systems might need multiple relays and actuators. |
| Power Dependent | No battery backup. A power failure disables all safety monitoring. |

# SECTION 11: Future Improvements

| Improvement | How to Implement | Benefit |
| --- | --- | --- |
| WiFi Alerts | Use ESP32 WiFi with Blynk/Telegram bot API to send smartphone notifications | Alerts even when user is away from home |
| Mobile App | Use ESP32 BLE + MIT App Inventor or Flutter app to display sensor data | Live monitoring on phone |
| Better Gas Sensor | Replace MQ135 with MQ-2 (LPG specific) or MQ-7 (CO specific) | Accurate gas type identification |
| Smoke Detection | Add MQ-2 or MQ-9 for better smoke/fire correlation with flame sensor | Fewer false alarms |
| Voice Alerts | Add DFPlayer Mini module with speaker for voice announcements | More intuitive and accessible |
| Data Logging | Add SD card module or use ESP32 SPIFFS to log sensor readings with timestamps | Review past incidents |
| Battery Backup | Add 18650 Li-ion battery + TP4056 charging module + boost converter | Works during power failures |
| Multiple Relays | Use 4-channel relay to control multiple fans, water sprinklers, alarms | Scalable industrial use |
| OLED Display | Replace LCD with 0.96" OLED for compact design and better visuals | Cleaner and smaller design |
| Cloud Dashboard | Connect to Firebase or ThingSpeak for real-time cloud monitoring and graphs | Professional monitoring system |

# SECTION 12: Viva Questions with Answers

Q1. What is the working principle of the MQ135 gas sensor?

The MQ135 is a chemo-resistive sensor. It contains a tin dioxide (SnO2) heating element. In clean air, oxygen ions adsorb on the surface creating high resistance. When gas molecules react with the adsorbed oxygen, they release electrons, reducing resistance. The ESP32 reads this resistance change as a varying analog voltage on GPIO34.

Q2. Why is the flame sensor active-LOW? What does this mean?

Active-LOW means the sensor outputs a digital LOW (0V) when it detects a flame, and HIGH (3.3V) in normal conditions. This is a common design choice to reduce power consumption in standby and to simplify certain circuit designs. In our code, we check: if (flameVal == LOW) to detect fire.

Q3. Why do both MPU6050 and LCD share the same two I2C wires?

The I2C protocol is a multi-device bus protocol. Each device has a unique 7-bit address. MPU6050 has address 0x68 and the LCD backpack has address 0x27. The ESP32 communicates with each one separately by sending the respective address before each data transfer. Multiple devices share SDA and SCL lines safely.

Q4. What is the purpose of the 220Ω resistors with LEDs?

LEDs have very low internal resistance. Without a series resistor, excessive current flows and immediately burns out the LED. With the 3.3V GPIO output and 220Ω: Current = (3.3V – 2V drop) / 220Ω ≈ 5.9mA. This is safe for both the LED (rated ~20mA max) and the ESP32 GPIO pin (rated 40mA max).

Q5. What is the difference between analog and digital sensor outputs? Which does MQ135 use?

Digital output gives only two values: HIGH or LOW (0 or 1). Analog output gives a continuous range of voltage representing the measurement precisely. MQ135 has both, but we use the A0 (analog) output on GPIO34 because it gives us exact gas concentration values (0–4095), allowing us to set fine-tuned thresholds and display the actual level on LCD.

Q6. Why is a relay module used instead of connecting the fan directly to the ESP32?

The ESP32 GPIO pins can only source a maximum of 40mA at 3.3V — far too weak to power a motor or fan. A relay is an electromechanical switch: a small ESP32 signal (few mA) activates the coil, which closes a separate high-current contact that can switch loads of 10A at 250V AC or 30V DC — safely powering the fan from its own power source.

Q7. How does gesture detection differentiate between tilt and shake?

Tilt is a slow, gradual change in the static acceleration values on one axis (gravity shifts direction). Shake is a rapid, simultaneous large change on multiple axes (sudden acceleration in random directions). We detect shake by comparing current vs previous readings: if the difference (delta) on any axis exceeds 15000 in one loop cycle (~200ms), it is classified as a shake.

Q8. What are the I2C addresses of the MPU6050 and LCD, and how are they set?

MPU6050: address 0x68 (when AD0 pin is connected to GND) or 0x69 (when AD0 is connected to 3.3V). This allows two MPU6050s on the same bus. 16x2 LCD with I2C backpack: usually 0x27 or 0x3F depending on the backpack model. The address is set by solder jumpers on the PCB. You can scan all I2C addresses using an I2C scanner sketch.

Q9. What safety precautions should be taken during this project?

Never test with actual LPG cylinder gas — use matchsticks or sanitizer vapor safely. Always use current-limiting resistors with LEDs. Do not connect 5V to MPU6050 (only 3.3V). Keep the relay's mains-side wiring away from the low-voltage ESP32 side. Allow MQ135 30 seconds warm-up. Never short circuit the ESP32 GPIO pins.

Q10. How can this system be upgraded to send mobile alerts?

The ESP32 has built-in WiFi. Using the WiFi.h and HTTPClient.h libraries, we can connect to a home router and send an HTTP request to a Telegram Bot API or Blynk IoT platform. When gas or fire is detected, the code sends a push notification to your smartphone — alerting you even when you are in another room or outside.

Q11. Why is there a priority order in the code (Emergency > Fire > Gas > Safe)?

In emergency situations, the most dangerous condition must override lesser ones. Fire is more immediately life-threatening than gas, so it has higher priority. Emergency mode (manual trigger) overrides all sensor readings to ensure a human can always force the maximum response. Without priority logic, conflicting conditions might produce an inconsistent or confusing system response.

Q12. What does the relay's NO (Normally Open) terminal mean?

NO stands for Normally Open. When the relay is not activated (coil is OFF), the NO terminal is disconnected from COM — the circuit is open and the fan is OFF. When the relay activates (coil is ON), NO connects to COM — the circuit closes and the fan turns ON. This is the safest configuration: if power is lost to the relay, the fan turns OFF by default.

# CONCLUSION

This Gesture-Controlled Smart Gas, Fire Safety and Ventilation System successfully demonstrates the integration of multiple sensors, gesture-based control, and automated safety responses using the affordable and powerful ESP32 microcontroller.

The system provides a practical, real-world safety solution that is both innovative — through touchless gesture control — and functionally robust — through dual-layer gas and fire detection with automated ventilation. It strikes an excellent balance between complexity and accessibility, making it ideal for student projects while demonstrating concepts relevant to real industrial IoT safety systems.

This project covers: Embedded Systems • I2C Communication Protocol • Sensor Interfacing • Interrupt-driven Design • Real-time Safety Systems • Gesture Recognition • Relay Control • IoT-ready Hardware

End of Documentation